PROM file images:

TDS307.INT     TDS2020 ANS Forth version  3.07    in Intel hex format

TDS307.MOT     TDS2020 ANS Forth version  3.07    in Motorola S-record format

TDS307A.INT    Alternative Forth ROM      3.07A   in Intel hex format

TDS307A.MOT    Alternative Forth ROM      3.07A   in Motorola S-record format

The files listed are images in PROM programmer format of TDS2020 ANS Forth

ROMs. You can make a new Forth ROM by downloading the appropriate file to a

PROM programmer (for example using the F12 key in TDS-PC for Windows).

Each file represents 4000 (hex) bytes of microprocessor code. Place it as

follows:

In a 27C128 EPROM:          whole device     from address (hex) 0000 to 3FFF,

use it in a TDS2020DV piggy-back board on the TDS2020 card.

In a 27C256 EPROM:          top    of device from address (hex) 4000 to 7FFF,

use it in a TDS2020DV piggy-back board on the TDS2020 card.

In a H8/532 microprocessor: bottom of device from address (hex) 0000 to 3FFF